| No. | Title | Authors / Institution | Year | Type | Main Idea / Contents | Relevance to our Work | PDF / Link |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ROS2 Multi-Zone Navigation with Fiducials | ETH Zurich (Student Labs) | 2022–2023 | Academic Project | Uses fiducial markers (AprilTags) to detect zones and trigger dynamic map switching in ROS2 | Practical example of event-based map switching in ROS2 | Institutional / Internal report |
| 2 | Multi-Map Navigation for Multi-Floor Buildings in ROS2 | University of Bonn | 2021–2022 | Academic Project | Maintains separate maps per floor; switches maps based on elevator/stair context | Supports context-aware and hierarchical map switching | Institutional / Internal report |
| 3 | ROS2 Map Tiling for Large Industrial Hall Navigation | Technical University of Munich (TUM) | 2023 | Master’s Thesis | Divides large industrial environments into multiple map tiles for scalable navigation | Closest match to your tile-based map switching approach | Institutional / Internal thesis |
| 4 | Multi-Map Localization for Long-Term Autonomous Navigation | Burgard Group, TUM | ~2018–2020 | Research / Thesis | Uses multiple local maps instead of one global map; switches maps using localization confidence | Core theoretical basis for tiled map switching | https://mediatum.ub.tum.de/doc/1751635/1751635.pdf |
| 5 | Experience-Based Navigation for Long-Term Localisation | Churchill, Newman (Oxford) | 2013 | Journal (IJRR) | Stores multiple “experiences” of the same environment; selects the most relevant map | Justifies multi-session and multi-map switching | https://journals.sagepub.com/doi/10.1177/0278364913499193 |
| 6 | FAB-MAP: Probabilistic Localization in the Space of Appearance | Cummins, Newman | 2008 | Journal / Conference | Appearance-based place recognition for selecting known regions | Useful for map identification before switching | https://www.robots.ox.ac.uk/~pnewman/papers/IJRRFabMap.pdf |
| 7 | Real-Time Loop Closure in 2D LIDAR SLAM (Cartographer) | Hess et al. (Google) | 2016 | Conference | Introduces submap-based SLAM with independent local frames | Strong justification for submaps / map tiles | https://research.google.com/pubs/archive/45466.pdf |
| 8 | Multi-Session Visual SLAM for Illumination-Invariant Re-Localization | Labbé, Michaud | 2022 | Journal | Maintains multiple maps under varying illumination and sessions | Supports switching between map sessions | https://pmc.ncbi.nlm.nih.gov/articles/PMC9243577/ |
| 9 | Lifelong Feature-Based Mapping in Semi-Static Environments | Rosen, Mason, Leonard | 2018 | Research | Addresses map aging, replacement, and long-term updates | Justifies maintaining multiple maps over time | https://research.google.com/pubs/archive/44821.pdf |
| 10 | Persistent Localization and Lifelong Mapping (FME) | Krajník et al. | 2017 | Journal | Introduces temporal persistence models in occupancy grids | Helps robust map selection over time | https://lcas.lincoln.ac.uk/nextcloud/shared/datasets/automated/paper.pdf |
| 11 | Map Segmentation for Robotic Navigation | Mielle | 2018 | Thesis | Techniques for segmenting large maps into regions | Basis for automatic tile generation | https://www.diva-portal.org/smash/get/diva2:1237531/FULLTEXT01.pdf |
| 12 | Image-Processing-Based Map Segmentation for Coverage Robots | Si et al. | 2020 | Journal | Image-processing methods to partition occupancy grids | Supports splitting ROS PGM maps into tiles | https://www.researchgate.net/publication/387818836 |
| 13 | Integrating Topological and Metric Maps for Navigation | Thrun et al. | 1998 | Research | Combines topological graphs with metric occupancy grids | Foundation for tile connectivity and transitions | https://www.cs.cmu.edu/~motionplanning/papers/sbp_papers/integrated1/fox_integrating_topo.pdf |