🔹 Research Paper 1 – ColMap (Memory-Efficient 3D Occupancy Mapping)

Purpose – Why the Method is Adapted

ColMap was proposed to address excessive memory consumption in high-resolution 3D occupancy grid mapping. A 100m × 100m × 30m environment at 5 cm resolution requires approximately 9.6 GB of memory, which is infeasible for embedded systems. The method exploits vertical structural continuity by applying run-length encoding along the vertical axis while using a 2D N² tree horizontally.

The goal was to achieve order-of-magnitude memory reduction without compromising probabilistic occupancy modeling.

Hardware Details

- Dataset captured using Velodyne Puck LiDAR mounted on UAV

- Motion capture system (48 OptiTrack cameras)

- Evaluation platform: Intel i7-5500U (2.4 GHz), 64-bit system

- Single-core execution

Benchmark Metrics

- Memory reduction: 12× lower than OctoMap (16-bit case)

- 20–40× lower than full occupancy grid

- Scan insertion: ~4× faster than OctoMap

- 1M cell access time comparable or better than OctoMap

Switching Cue and Algorithm

No map switching mechanism is implemented.
ColMap maintains a single compressed global map using:

- Log-odds Bayesian update

- Vertical run-length compression

- N² tree spatial indexing

Comparison with Our Proposal

ColMap reduces memory from gigabytes to hundreds of megabytes, but memory still scales with environment size.

Our tile-based approach reduces active costmap from:

- ~1.6 million cells (≈60m × 45m @ 0.04m resolution)
tol

- ~50,000–150,000 cells per tile

Memory becomes constant regardless of total environment size.

ColMap compresses one global map.
Our method avoids storing the global map entirely.

🔹 Research Paper 2 – OctoMap

Purpose

Provide a probabilistic 3D volumetric mapping framework using an octree data structure to efficiently represent free, occupied, and unknown space.
The method aims to reduce memory consumption compared to full 3D grids while supporting multi-resolution queries and scalable mapping in large indoor and outdoor environments.

Hardware

- SICK LMS, Velodyne, Kinect datasets

- Intel i7-2600 (3.4 GHz)

- 32-bit architecture used for memory evaluation

Benchmark Metrics

Freiburg Campus dataset (10 cm resolution):

- Full 3D grid: 5162.90 MB

- Octree (pruned): 990.66 MB

- Maximum-likelihood compressed: 504.76 MB

Mapping accuracy: 97–98%

Traversal time for 4M leaf nodes: ~51 ms

Switching

No switching implemented.
Maintains a single dynamically growing octree.

Comparison

Even after compression to ~504 MB, OctoMap still scales with environment size.

Our approach ensures memory bounded to a single tile (~50k–150k cells), making memory independent of global environment size.

OctoMap focuses on 3D probabilistic mapping.
Our system operates at the 2D navigation layer with deterministic map replacement.

🔹 Research Paper 3 – BOBMapping

Purpose

BOBMapping was designed to enable fully autonomous indoor mapping on extremely resource-constrained robotic platforms with very limited onboard memory. It uses probabilistic occupancy representation with a binary 1-bit-per-cell model rather than traditional occupancy grid which uses floating points. In these cells are CLASSIFIED simply as free or occupied. During mapping, minimal additional bits are temporarily used for exploration logic, and the final stored map remains highly compact. The goal was to drastically reduce memory requirements and computational overhead while still achieving reliable mapping accuracy for small indoor environments.

Hardware

- FPGA Spartan3-1000

- 32 kB internal RAM

- 512 kB external RAM

- 16×15×18 cm robot

- IR sensors

Benchmark Metrics

- Mapping accuracy: 99.42%

- Final map storage: 1 bit per cell

- Test grid: 11×14 cells

- Memory usage: 28 bytes final map

Switching

No switching.
Uses a fixed small grid.

Comparison

BOBMapping handles extremely small environments (11×14 cells).

Our environment:

- 60m × 45m

- ~1.6M cells at 0.04m resolution

Our solution scales to building-level navigation.

🔹 Research Paper 4 – RC-Map

Purpose

Reduce memory usage and path planning computation time by converting occupancy grid maps into rectangular region-based graph representations.
The method aims to compress free space into larger rectangular nodes, enabling faster path generation and reduced turn complexity compared to cell-based grid planning.

Hardware

Embedded board evaluation (simulation-based).

Benchmark Metrics

- Memory usage: 8768 bytes (~3% of equivalent 300×300 grid)

- Path planning time: 10–15 ms

- Distance Transform: 740–1238 ms

- Turns reduced from 40 to 6

Switching

No switching.
Single abstracted map.

Comparison

RC-Map reduces memory to 3% of grid map but still keeps entire map active.

Our method reduces active map size from 1.6M cells to 100k cells (~94% reduction in active search space).

🔹 Research Paper 5 & 11 – Semantic Zone-Based Map Management

Purpose

The semantic zone-based map management approach was proposed to address the working memory instability and memory explosion issues observed in dense 3D SLAM systems operating in large indoor environments such as hospitals. Instead of having a continuously growing Key point values, the environment is partitioned into semantically meaningful zones (e.g., corridors, rooms, lobbies), and memory management is performed at the zone level rather than at the individual keyframe level. When the robot transitions between zones, relevant zone data is loaded into working memory while less relevant zones are transferred to long-term memory. The goal was to stabilize memory usage, reduce redundant load/unload operations, and improve long-term navigation performance without discarding the global SLAM map structure.

Hardware

- NVIDIA Isaac Sim

- RTAB-Map

- RGB-D camera

- 2D LiDAR

Benchmark Metrics

- Reduced unnecessary load/unload cycles

- Enforced memory threshold Mmax

- Improved working memory stability

(No specific MB reduction provided.)

Switching Algorithm

Switch triggered when robot enters zone boundary.
Memory prediction:

If predicted memory > Mmax
→ Remove least recently used zone

Multiple zones may coexist.

Comparison

Semantic zone methods still maintain entire global database.
Multiple zones remain active simultaneously.

Our system:

- Only 1 tile active at a point in time

- Complete map_server  and costmap reload on switch

- Old map fully offloaded memory

- Heading validation to avoid unnecessary switching

Memory remains strictly bounded.

🔹 Research Paper 6 – Grid-Based Submap Joining for Large-Scale 2D SLAM

Purpose
Improve SLAM scalability by dividing large environments into local submaps and optimizing their merging using a Nonlinear Least Squares (NLLS) formulation.
The method aims to reduce computational cost while maintaining global consistency and high pose accuracy in large environments (up to 250m × 200m).

Hardware
• Intel Core i7-1370P
• Evaluated on large-scale datasets (up to 250m × 200m)
• Desktop-class computation platform

Benchmark Metrics
• 3–12× smaller pose error than Cartographer
• Submap joining time: 2–6 seconds (large datasets)
• 160–280% faster per iteration using pose-only optimization
• Demonstrated scalability on large museum datasets

Switching
No runtime switching.
Submaps are generated locally but eventually merged into a single global occupancy map.
All submaps remain stored in memory.

Comparison
Submap joining improves optimization efficiency but memory still grows with environment size.
All submaps are retained, meaning:

Memory ∝ environment size.

Our method avoids global map growth entirely.
Only one tile (~50k–150k cells) is active at any time.
Memory remains constant regardless of total explored area.

🔹 Research Paper 8 – Pole Landmark-Based Long-Term Localization

Purpose
Improve long-term localization robustness in large urban environments by replacing dense 3D maps with sparse, structurally stable pole landmarks extracted from LiDAR scans.
The method aims to reduce memory requirements and improve resilience to seasonal and structural changes while maintaining sub-meter localization accuracy over long-term datasets (15 months).

Hardware
• Velodyne HDL-32E LiDAR
• NCLT Dataset (15 months of data collection)
• GPS, IMU, wheel encoders
• Evaluated on desktop-class computing systems

Benchmark Metrics
• Mean localization accuracy: 0.284 m
• 54% reduction in position RMSE
• 69% reduction in heading RMSE
• Global map contained ~1889 poles

Switching
No switching mechanism implemented.
Maintains a single global landmark map.
Full landmark database remains active during runtime.

It is Good At

✔ Localization
✔ Loop closure
✔ Long-term consistency

It Cannot be used in :

- Path planning

- Obstacle avoidance

- Collision checking

- Costmap generation

Comparison
The method reduces map density but still maintains one global landmark database.

Memory ∝ number of landmarks ∝ environment size.

Our approach does not sparsify the map.
Instead, it dynamically replaces entire occupancy grid tiles.
Only one tile remains active at runtime.
Memory remains strictly bounded and independent of global environment size.

🔹 Research Paper 9 – Utilizability of Navigation2/ROS2 in Industrial Systems

Purpose
Validate the deployment of the ROS2 Navigation2 (Nav2) stack in industrial robotic systems using layered costmaps and Behavior Tree-based control.
The method aims to ensure deterministic navigation, real-time replanning, and robust obstacle handling in structured industrial environments while maintaining modular system architecture.

Hardware
• Raspberry Pi 4B (low-level motor control)
• Intel NUC (high-level navigation computer)
• Velodyne HDL-32E LiDAR
• Differential drive robot

Costmap configuration:
• Global costmap resolution: 0.05 m
• Global update rate: 1 Hz
• Local costmap resolution: 0.025 m
• Local rolling window: 3 × 3 m
• Local update rate: 5 Hz

Benchmark Metrics
• Successful dynamic replanning in industrial scenarios
• Global costmap update: 1 Hz
• Local costmap update: 5 Hz
• Recovery behaviors validated (clear, spin, back up)

Switching
No tile-based switching implemented.
Uses a single static global map with layered costmaps.
Local costmap operates as a rolling window but does not replace the global map.

Comparison
The system uses Nav2 exactly as designed with a static global map.
Map size still scales with environment size.

Our system extends Nav2 by:
• Adding a custom Behavior Tree plugin (TileCheckAction)
• Calling map_server load_map dynamically
• Clearing costmaps during transitions
• Using 2–4m overlap trigger zones
• Applying ±45° heading validation
• Ensuring only one tile is active

Our approach transforms Nav2 from stat

Final Overall Comparison Summary

Across all 11 papers:

- Memory reduction achieved: 3%–40× compression

- Map sizes still scale with environment

- No deterministic geometric overlap-based switching

- No heading-validated switching

- No navigation-layer full map replacement

Our system uniquely:

- Reduces active costmap from ~1.6M cells to ~50k–150k cells

- Guarantees constant memory

- Uses 2–4m overlap trigger zones

- Uses ±45° yaw heading validation

- Integrates switching inside Nav2 Behavior Tree

- Fully replaces map via map_server