A Novel Map Switching Algorithm and Generation Pipeline for Autonomous Mobile Robot localization in Eccentric Scenarios

Introduction (Cite as many articles as possible)

Maps for indoor localization

Types

SOTA

Best Practices

Challenges

Tradeoff between size and resolution

Strategies to address  All methods to be surveyed

Comparison of available approaches

Table and Diagram

Experimental Setup and Software Tools

Robot - Dani robot chassis (none of it’s electronics so ig doesn’t make much value), a jetson nano of 4GB (the computer on the robot, but we also test a lot with my laptop since it’s a containerized docker environment, so the differentiating point would be the hardware), we use Zed Mini with zed sdk 4.2.5, in terms of software we use zed sdk 4.2.5, ros2 humble on ubuntu 22, and nav2 for this particular task, for localizaing we use the zed camera’s inbuilt algo which is part of zed wrapper and can enable with zed config file, we have turned on imu fusion along with visual odometry from it. The imu is inbuilt into zed cam and zed sdk and wrapper handles it.

Sensors – just zed mini, didn’t externally use any sensors directly tho the sdk uses all of it ig

Application – so the task was more like we have to operate the robot in our college corridor (hi tech second floor of my uni (srmist, ktr)). And when we built the map we observed that since the robot should operate only in the corridoors most of the region on the map was black meaning we consider as an obstacle just that it doesn’t need to go there at all, but the main part this made it that most of the memory would just be used for storing parts of the map that wouldn’t be used it’s more like a square with inbetween a portion is the usable root inside and outside is unusable. So we split the map into segments such that the portion usable is dominant, and now most memory is used for usable area and overall the the memory area itself reduces even if that problem isn’t there cause we only load smaller portions in the memory and costmaps are built upon only that area so a cumulative upgrade, also it give an advantages of keeping it more scalable cause you need not modify the whole map if anything changes or you need to add a new portion, just add or modify the tile alone keeping it highly scalable too. I mean this is the thing but I think this isn’t the application ig, idk do we talk about the tile switch application or robot itself, so if it’s about the robot the goal is more like a floor level material transfer robot, like just transfer files n stuff between cabins and classrooms within the floor so a human need to be used here putting human resource to better more useful tasks than something brainless.

Requirements in terms of parameters quantified
hmm not sure what exactly this means, but we classified cpu usage and memory across different servers of nav2 on a comparison basis with using the full tile and broken tile (we call it tessalted)

Libraries used
libraries you mean for tile switching right? I’ll give you the code base you can check it out

Nomenclatures
not sure what exactly you mean here

Map Data Generation Pipeline

Process

Measurement

CAD

CAD to Image

Image to Map

Map as a data structure

Scaling issues

Tessellation of the map

Specification of the map

Comparison of the tessalated map with whole map

Compare resolution, computational overheads, map update time, memory

Numericals, Tables, plots.graphs

Map Switching Algorithm

Cues for switching

Switching artifacts and inaccuracies

Use of external fiducials for better localization during switching

Compare localization accuracy with and without aruco marker in place