1. ETH Zurich – ROS2 Multi-Zone Navigation with Fiducials (2022, 2023 student labs)

Lab: ETH Zürich Autonomous Systems Lab
System: Turtlebot4 + Nav2
Method:

- Large campus corridors divided into 6 ROS2 tiles.

- AprilTags placed in doorway boundaries.

- Tile switching triggers when AprilTag ID detected.

- /initialpose published to AMCL after switch.

Why they chose map switching

- Global map exceeding memory limits on Turtlebot4 SBC (large corridor map at 0.02 m resolution was ~70MB).

- Costmap update slowdown — Nav2 global_costmap became too slow on large maps.

- Desire for high-resolution local maps without making full global map huge.

Difficulties faced

- AMCL instability when switching maps → particle reset required careful covariance tuning.

- Timing issues: AMCL sometimes read /initialpose before map update, causing incorrect localization.

- Tile edge discontinuity: robot would see walls slightly shifted between tiles unless pixel alignment was perfect.

- AprilTag false detections in strong sunlight inside glass corridors.

- Nav2 global planner sometimes failed when a switch occurred mid-plan.

2. University of Bonn – Multi-Map Navigation for Multi-Floor Buildings in ROS2 (2021–2022)

Project: BONN ROS2 Campus Robot
Platform: Summit XL robot
Technique:

- Multi-floor building map split into per-floor submaps.

- QR codes & AprilTags at staircases and corridor ends.

- ROS2 Nav2 used.

- Detect tag → switch to floor/map → reseed AMCL → continue path.

Why they chose map switching

- Different floors had different levels of detail.

- Loading full building as one map exceeded Nav2 costmap limits (global costmap area too large).

- Wanted to avoid dynamic map stitching, so static submaps were easier.

Difficulties faced

- Localization drift before tags, requiring larger overlap zones.

- Switching mid-navigation caused global planner resets → robot briefly stopped.

- AMCL took time to converge after reseeding, so motion needed to be slowed for ~1 second.

- Map origins had to be aligned extremely accurately across floors, otherwise pose jumps occurred.

3. Technical University of Munich – ROS2 Map Tiling for Large Industrial Hall Navigation (2023 Master Thesis)

Title (translated): "Sub-Map Based Navigation Using ROS2 for Large Industrial Environments"
Platform: MiR-like indoor mobile robot
Technique:

- Industrial hall (~80m × 30m).

- Global map too large for onboard Cortex-A53 computer.

- ROS2 Nav2 used with 4 tiled maps.

- ArUco markers placed at transitions.

- Tile loaded via custom map_manager node → publishes to /map.

- AMCL reinitialized every switch.

Why they chose map switching

- Memory constraint: full-resolution map consumed too much RAM.

- AMCL performance dropped severely on huge maps (particle count required became too large).

- Wanted high-resolution obstacles around machines, impossible with single-map design.

Difficulties they documented

- Nav2 costmaps got confused during map replacement → needed a hack to clear costmaps after switch.

- Camera extrinsic calibration errors led to wrong AMCL initial pose after tag detection.

- Robot sometimes triggered switch too early → required hysteresis & multi-frame confirmation.

- Path smoothing broken when switching mid-path → global planner needed forced recompute.

| Project | Why Map Switching | Major Difficulties |
| --- | --- | --- |
| ETH Zürich ROS2 Multi-Zone | Map too large, need high-res local details, SBC memory limits | AMCL timing, tile misalignment, AprilTag noise |
| University of Bonn Multi-Floor ROS2 Robot | Multi-floor maps, Nav2 costmap too large, need modular tiles | Pose jumps, reseeding delays, planner resets |
| TUM Industrial Hall ROS2 Navigation | Huge environment, AMCL slowdown, high-res around machines | Costmap corruption, camera calibration errors, early switches |