COMPLETE ROS2 MAP SWITCHING PROCESS

In ROS2, map switching is not built-in — we must override the /map topic by publishing a new nav_msgs/OccupancyGrid message containing the new tile map.
Every Nav2 component subscribed to /map will automatically receive the new map (if QoS is set correctly).

Tile switching works because:

- we publish a new map on /map → AMCL + global_costmap immediately use it.

- Then you publish a new /initialpose → AMCL resets to correct pose based on ArUco marker.

# STEP1:NODES RUNNING

- The system consists of these major nodes:

| Node | Role |
| --- | --- |
| Nav2 Map Server (optional) | Usually publishes static map, but in switching method YOU replace it |
| Nav2 AMCL | Localizes robot using /map and laser scan |
| Nav2 Controller / Planner / BT Navigator | Performs global path planning and movement |
| Your Tile Switcher Node | Detects ArUco + publishes new tile map + resets AMCL |
| Camera Node | Publishes /camera/image_raw |
| TF2 | Provides transforms (base_link, camera_link) |
| LIDAR Node | Publishes /scan |
| Costmap Server(s) | Build global & local costmaps from /map |

STEP 2:TOPIC SUBSCRIPTION AND PUBLICATION

Topic Subscriptions and Publications (Critical Part)

Tile Switcher Node subscriptions

Tile Switcher Node publications:

STEP 3:

What ROS2 modules do with /map

AMCL subscribes to /map

- The moment you publish a new map on /map, AMCL receives it.

- AMCL internally resets and reconstructs likelihood fields based on the tile.

BUT AMCL still doesn’t know where the robot is.
That's why you must publish /initialpose.

Nav2 Costmap Server subscribes to /map

- Global costmap rebuilt using the tile

- Local costmap anchored to new map frame

- No code changes needed → costmap automatically updates

Nav2 Planner Server uses /map indirectly through costmaps

When map changes:

- Planner server will receive a new global costmap

- It can replan the global path automatically

STEP 4: SWITCHING SEQUENCE

STEP 1 — ArUco detected

Camera publishes → Tile switcher node receives image_raw

Tile Switcher:

- Detect marker corners (OpenCV aruco)

- SolvePnP → get marker pose in camera frame

- Apply TF camera→base_link → compute robot pose in map frame

- Identify which map tile to load next based on marker ID

STEP 2 — Tile Switcher publishes NEW MAP

STEP 3 — ROS2 Nav2 modules react

AMCL receives NEW /map

- AMCL clears old likelihood fields

- Recomputes map model

- Laser scan matching resets

Global Costmap Server receives new /map**

- Old obstacles cleared

- New costmap built from tile

- Boundaries, walls, corridors updated

Planner Server sees new /map via costmap**

- Global planner becomes aware of new environment

- It will need a new goal or forced replan

STEP 4 — Tile Switcher publishes /initial pose

- position (x, y, yaw) = pose computed from ArUco

- covariance tuned for safe AMCL initialization

AMCL sets the new robot pose in the new tile

STEP 5 — Nav2 Behavior Tree restarts navigation

- BT Navigator reads updated pose (from AMCL)

- Global planner replans path in new tile

- Controller follows new local plan

ROS2 Tile Switching Process (Full Flow) summary:

- Camera Node publishes /camera/image_raw.

- Tile Switcher Node detects an ArUco marker at a tile boundary.

- Tile Switcher computes robot pose from the detected marker using solvePnP and camera→base TF.

- Tile Switcher publishes the new tile map to /map with TRANSIENT_LOCAL QoS.

- AMCL, Global Costmap Server, and Planner Server immediately update because they subscribe to /map.

- Tile Switcher publishes /initialpose containing pose computed from ArUco.

- AMCL resets particle filter using /initialpose and aligns robot to the new tile.

- Tile Switcher calls Nav2 costmap clear services to remove old tile costmaps.

- Nav2 Behavior Tree (BT Navigator) triggers global replanning using the new map tile.

- Robot continues navigation inside the new map region.